${QR}

Attention to: ﻣﺆﺳﺴﺔ ﻋﻨﻴﻔﻪ ﺳﻠﻴﻤﺎﻥ ﺑﻦ ﻋﻠﻴﺎﻥ ﺍﻟﺮﺷﻴﺪﻱ ﻟﻠﻤﻘﺎﻭﻻﺕ ﺍﻟﻌﺎﻣﺔ

IDI PR #: 123111

Established by: Amr Yasser

# TIS Visit

# Inspection Report - Retaining walls

Date of visit: 27-Jan-2025

Location: 26.4355518,50.1022324

( Special Projects, Saudi Arabia)

| Owner | ﺿﻴﻒ ﺍﻟﻠﻪ ﺑﻦ ﻣﺤﻤﺪ ﺑﻦ ﻣﺜﻌﻲ ﺍﻟﻌﺘﻴﺒﻲ | TIS Company | CPV Arabia |
| --- | --- | --- | --- |
| Contractor | ﻣﺆﺳﺴﺔ ﻋﻨﻴﻔﻪ ﺳﻠﻴﻤﺎﻥ ﺑﻦ ﻋﻠﻴﺎﻥ ﺍﻟﺮﺷﻴﺪﻱ ﻟﻠﻤﻘﺎﻭﻻﺕ ﺍﻟﻌﺎﻣﺔ | Report Issue Date |  |
| Project Location | ﺍﻟﻘﻄﻌﺔ ﺭﻗﻢ 701 ﻣﺨﻄﻂ 92*6 ﺍﻟﻤﻨﺘﺰﺓ ﺭﺍﺱ ﺗﻨﻮﺭ | Inspection # | 1 |
| Inspection Stage | Retaining walls | No. Of buildings | 1 |
| Inspector Name | Amr Yasser | Work in progress | Retaining walls |
| Email | ayasser@cpvarabia.com | Telephone | 594037209 |

Inspection Results: Satisfactory

Description of the inspections carried out: CPV ARABIA has conducted an on-site technical inspection (IDI) for Project PR #123111 during the pre-pouring phase of the Retaining walls . The details of the inspection are described further as we go through this report.

Summary

Risks Assessment: - The load is applied incrementally not as per approved standards (e.g., ASTM D1196). the load is applied incrementally should comply the approved standards (e.g., ASTM D1196). - The compaction test results for the previous layers have not been provided and have not been verified on-site The compaction test results for the previous layers must be provided and verified on-site. Stages missed without TIS involvement: - Not exist Interpretation of Additional Visit: - The test area is not level, not free from debris, and not prepared according to the approved method statement

General Site Pictures and Construction Plans

General site picture and construction plans

Checklist

| No | Checklist | SBC Reference | Result | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Inspection criteria |  |  |  |
| 1.1 | Is the site ready for inspection with safe inspection conditions and safe access for the TIS inspection? | - | Passed |  |
| 1.2 | Were there any modifications in the project that is different from the RD0, Soil Report and Construction Plans...etc.)? | - | Passed |  |
| 1.3 | Is there any missing inspection stages in the project? | - | Passed |  |
| 1.4 | Are there any defects observed that can affect the building’s stability or RD5 inspection for slab stage hasn’t been closed yet? |  | Passed |  |
| 1.5 | Has the client provided verified concrete compressive strength test results? |  | Passed |  |
| 1.6 | Is the laboratory conducting tests certified by an ISO/17025-accredited body? | - | NA |  |
| 2 | Formwork |  |  |  |
| 2.1 | Are the forms clean and ready for pouring concrete? | - | Passed |  |
| 2.2 | Is the formwork assembly stable due to the different loads? | - | Passed |  |
| 2.3 | Were Concrete covers maintained as per approved drawings? | - | Passed |  |
| 3 | Steel Reinforcement |  |  |  |
| 3.1 | Are the longitudinal reinforcement diameter and number executed as per the construction drawing? | - | Passed |  |
| 3.2 | Was the Water-stop installed as per the drawings? | - | Passed |  |
| 3.3 | Is the length of splices according to the SBC specifications? | - | Passed |  |
| 3.4 | Is the anchorage length of steel reinforcement at the corners of the retaining walls according to the SBC specifications? | - | Passed |  |
| 3.5 | Is the steel reinforcement free from rust, oil, and contaminants? | - | Passed |  |
| 3.6 | Are the tie wires installed as required? | - | Passed |  |
| 3.7 | Are the bends bars within the radius and tolerance uniformly made? | - | Passed |  |
| 3.8 | Is the thickness of the Retaining walls complying with the construction plans? | - | Passed |  |
| 3.9 | Do the diameter, quantity, direction, and spacing of the additional longitudinal reinforcement at intersections, openings, and corners comply with the plans? | - | Passed |  |
| 3.10 | Are the pipe sleeves installed as per engineering principles? | - | Passed |  |
| 3.11 | Are the horizontal reinforcement diameter and number executed as per the construction drawing? | - | Passed |  |

Risk Assessment

the horizontal reinforcement diameter and number are executed as per the construction drawing Passed

Additional steel reinforcement comply with construction plans Passed

Retaining wall thickness Passed

Bent bars Passed

The Anchorage length of the retaining walls steel reinforcement at corners Passed

The splice length Passed

Was the Water-stop installed as per the drawings? Passed

the longitudinal reinforcement diameter and number are executed as per the construction drawing Passed

Concrete Cover Passed

| Engineer In Charge of Inspection | Area Manager | Technical Inspection Manager |
| --- | --- | --- |
| Name: Amr Yasser | Name: | Name: |
| Signature: | Signature: | Signature: |
| Date of Issuing the report: |  |  |